# ANNEX II + III :	 TECHNICAL SPECIFICATIONS + TECHNICAL OFFER

Supply of electric bus and electric chargers within APOLLO project

01-908/24-122/17

Columns 1-2 should be completed by the Project partner

Columns 3-4 should be completed by the tenderer

Column 5 is reserved for the evaluation committee

Annex III - the Contractor's technical offer

The tenderers are requested to complete the template on the next pages:

- Column 2 is completed by the Project partner shows the required specifications (not to be modified by the tenderer),

- Column 3 is to be filled in by the tenderer and must detail what is offered (for example the words “compliant” or “yes” are not sufficient)

- Column 4 allows the tenderer to make comments on its proposed supply and to make eventual references to the documentation

The eventual documentation supplied should clearly indicate (highlight, mark) the models offered and the options included, if any, so that the evaluators can see the exact configuration. Offers that do not permit to identify precisely the models and the specifications may be rejected by the evaluation committee.

The offer must be clear enough to allow the evaluators to make an easy comparison between the requested specifications and the offered specifications.

The requirements set out in the technical specifications represent the minimum technical characteristics which offered goods must satisfy, unless stated otherwise, and tenderers are not allowed to modify technical specification in any way.

For each item for which it is not explicitly stated that it is allowed to offer goods of the equal characteristics, i.e. for each item where it is not stated “or equivalent”, for the purposes of this tender documentation it is assumed that words “or equivalent” are stated, and tenderer is allowed to offer equivalent goods / goods of equivalent characteristics.

| 1. Item no. | 2. Specifications Required | 3. Specifications Offered | 4. Notes, remarks, ref to documentation | 5. Evaluation Committee’s decision (Y/N) |
| --- | --- | --- | --- | --- |
| LOT 1: Electric bus |  |  |  |  |
| 1.1 | Electric bus – 1 piece Basic Technical Data Year of manufacture – min. 2024 Condition – newly manufactured Vehicle category – M3, Class A, low-floor city bus Electric minibus – min. 10 seats and 10 standing places Construction – steel monocoque frame or similar Wheelchair space – min. 1 space Maximum speed – limited to max. 70 km/h Vehicle color – white Vehicle branding – after contract signature based on contracting authority’s design Dimensions Length – 5700–6400 mm Width (without mirrors) – 2000–2300 mm Height – max. 3000 mm Permissible vehicle weight – max. 5500 kg Passenger compartment height at doors – min. 2000 mm Motor and Drive Drive type – electric Power – min. 125 kW (170 HP) Gradeability – min. 25% Fire detection – system for electric motor Torque – min. 290 Nm Transmission – automatic Energy recovery – regenerative braking system Batteries and Charging Battery – Li-ion 360 V, min. 88 kWh (NMC) Safety – battery fire detection system Range – min. 260 km (E-SORT 2 certified) Charging time – max. 250 min (DC), max. 6 h (AC) Emergency charging – built-in AC charger min. 22 kW Standards – IEC 61851, IEC 62196, ISO 15118, DIN 70121 Charging control – electronic monitoring system Charging connector – Type 2 and CCS2 Battery status – displayed to driver Auxiliary Battery Low-voltage battery monitoring and charging system (12V/24V) Suspension Front – independent, springs, telescopic shock absorbers Rear – independent, coil springs, anti-roll stabilizer Braking and Safety Hydraulic dual-circuit brakes, ABS, hill holder Disc brakes with wear indication Electric parking brake with automatic activation Regenerative braking integrated Equipment Audio system (radio, USB, Bluetooth) STOP buttons in passenger area USB chargers (min. 2) Front display (min. 1150×200 mm) Electric mirrors Rear camera and parking sensors Roof hatch Central locking (remote) Wipers (multi-speed) Sealed electrical connectors Hill holder Doors Electric double doors (glazed) Min. width 1100 mm Automatic control + emergency manual operation Audio signal on closing Lighting Interior LED lighting Exterior LED lights (daytime, headlights, rear, stop) Passenger Area Wheelchair space with signaling button Manual ramp (min. 300 kg capacity) Safety belts and holders Fixed passenger seats Driver Cabin Adjustable pneumatic seat Fully adjustable steering wheel Power steering Sun protection blinds Semi-closed cabin Heated windows Electric side window Storage compartments Climate and Heating Roof AC for passengers + driver AC Independent climate control for driver Temperature range 18–26°C Heating system for −15°C conditions Electric heating min. 12 kW Chassis Corrosion-resistant body panels Corrosion-protected steel structure Service panels with gas struts Towing hook Wheels and Tires Tire pressure monitoring system Max. 2 axles Aluminium wheels (min. 215/75/R16) Vehicle Operation Start/stop button Ignition key system Diagnostics Onboard diagnostic system with fault recording (date/time) Interface for diagnostic tools |  |  |  |
| LOT 2: Electric charger |  |  |  |  |
| 2.1 | Electric charger – 2 pieces  Stationary DC charger for electric vehicles – min. 30 kW output power  Charger type – stationary DC charger  Vehicle connector type – CCS  Input power – 3×400 V / 50 Hz  Condition and year of production – 2024 or newer, new and unused  Grid connection – direct connection to the distribution board (hardwired)  Maximum charging current – min. 100 A  Output voltage range – min. 150 V, max. 1000 V  Enclosure material – aluminium, steel or similar  Charging modes – IEC 61851-1, IEC 61851-23, IEC 61851-24, ISO 15118, DIN 70121  Protection rating – min. IP54 / IK10  Noise level – max. 60 dB  Minimum operating temperature – min. −20°C  Compliance with standards – CE, LVD 2014/35/EU, EMC 2014/30/EU, RED 2014/53/EU  Additional equipment – emergency stop button, LCD touchscreen display, GSM connectivity • Stationary DC charger installation, testing and certification in accordance with applicable national/local regulations |  |  |  |